# Research on Observation Space and Reward Function

# Observation Space

- Formally, the observation space is:

st=(ht,pt,dt,cosφtd,sinφtd)

- ht∈ RL is a vector of L tank levels at time t

- pt∈ RJ is a vector of J junction pressures at time t

- dt is the estimated total demand at time t

- φtd∈[0,2π] is the angular position of the clock in a day

- φd= 2πτdTd,  where τd∈[0,Td] denotes the time of day in seconds and Td is the total number of seconds in a day (86400).

- Although all tanks must be monitored, we can reduce dimensionality of the observation space by considering only junctions placed in strategic positions.

| Index | Count | Feature Name | Description |
| --- | --- | --- | --- |
| 0 - 1 | 2 | Tank Levels | Height of water in the 2 tanks (T41, T42). |
| 2 - 23 | 22 | Junction Pressures | Pressure sensors at all 22 nodes (J1...J22). |
| 24 | 1 | Avg Demand | Moving average of town water usage. |
| 25 | 1 | Time (Sin) | Cyclic time encoding (so 23:59 connects to 00:00). |
| 26 | 1 | Time (Cos) | Cyclic time encoding. |
| Total | 27 |  |  |

## Proposed Changes

Based on Research switch to using the exponential moving average (EMA):

  - EMA (Exponential Moving Average): This has "Recency Bias." The most recent data point gets the most weight, and the importance of older data fades away exponentially.

  - To calculate an SMA of the last 10 steps, you must store a list of the last 10 numbers. If you want a 24-hour history, you must add 24 new inputs to your Neural Network for every single sensor. This makes your observation space massive and hard to train.

  - EMA does not need a buffer. It only needs one number: the previous EMA value.

    - EMANEW=α×Current+((1-α)×EMAOLD)

    - α is the "Smoothing Factor" -> If α = 0.3, it means: "Take 30% of the new reality and keep 70% of history." You only add 1 new input per sensor, regardless of how long the history is.

### Case Scenario for Water Network Agent

- Imagine a pipe bursts in Anytown. The pressure at Junction J12 drops from 50psi to 10psi instantly.

      - Scenario A: Using SMA (Last 10 hours)

        - The SMA looks at the last 10 hours. 9 of those hours were "Normal" (50psi), and only the current one is "Bad" (10psi).

        - The SMA calculates: 50+50+50+50+50+50+50+50+50+1010=46 psi

        - The Agent thinks: "The pressure is 46. That's fine. I don't need to do anything." (The Agent is too slow).

- Scenario B: Using EMA (α = 0.5)

  - The EMA looks at the previous average (50) and the new value (10).

  - Calculation: 0.5× 10+0.5× 50=30 psi

  - The Agent thinks: "Whoa! Pressure dropped to 30! I need to turn on a pump immediately!" (The Agent reacts fast).

# Action Space

- The discrete action variable at∈N can assume values in {0,…,2P-1}, with P number of pumps within the system.

- The action determines the combination of open/closed pumps.

# Reward Function

- Formally, the objective is:

  - maxt=1T[(w1 × rDSRat)+(w2 × rofat)]

- Instantaneous reward given by the environment is

  - rt=(W1 × rDSR,tat)+(w2 × rof, tat)

- rDSR,tat=j=1Jdj,tj=1Jdj,t , where dj,t denotes the actual supplied demand at junction j and time t, and dj,t is the corresponding expected demand.

- This term rewards the agent proportionally to the fraction of demand satisfied, with rDSR,tat=1 indicating perfect demand satisfaction

- Reward Coefficients are defined in the parameter_generator method

- Individual reward components are stored in rewards.py and coefficients are stored in world_anytown.yaml

- Currently W1 = 1.0 and W2 = -0.25

Potential Improvements

Add the following components:

- Adding check_pumps_updates (Pump Switching Penalty)

    - Without this, RL agents tend to "chatter"—turning a pump ON in step 1, OFF in step 2, and ON in step 3 to chase tiny pressure gains. In the real world, this would burn out the pump motor in a week.

    - Weight (-0.05): This is a "soft constraint." We want to discourage switching, but if the town is about to run dry (DSR drops), the agent should feel free to switch the pump ON to save them.

  - Adding check_pumps_flow (Energy Proxy)

    - This function returns a normalized value (0 to 1) representing how much water you are moving. Generally, More Flow = More Energy = Higher Cost. By penalizing this, you teach the agent to be "lazy"—only pump exactly as much water as is needed to satisfy DSR, and not a drop more.

    - Weight (-0.1): Energy is important, but never more important than satisfying demand.

| Objective | Function | Weight | Reasoning |
| --- | --- | --- | --- |
| Reliability | dsr | +1.00 (w1) | The primary goal. If this is 0, nothing else matters. |
| Safety | overflow | -0.25 (w2) | Strong penalty. Prevents physical damage/flooding. |
| Efficiency | check_pumps_flow | -0.10 (w3) | Saves money. Encourages using gravity instead of pumps when possible. |
| Longevity | check_pumps_updates | -0.05 (w4) | Saves hardware. Prevents rapid oscillation (flickering). |

Formally, the target objective is:

maxt=1T(w1 × rDSRat+w2 × rofat+ w3 × rcpfat+ (w4 × rcpuat)]